Code:

/************************************************************

SMART HELMET - ADVANCED ACCIDENT ALERT SYSTEM

ESP32 + MPU6050 + TELEGRAM BOT

Impact + Tilt Confirmation + Dual Emergency Contacts

************************************************************/

#include <Wire.h>

#include <Adafruit_MPU6050.h>

#include <Adafruit_Sensor.h>

#include <WiFi.h>

#include <HTTPClient.h>

/* -------- WiFi Credentials -------- */

const char* ssid = "DESKTOP-DQD9QHP 3139";

const char* password = "6w5:30Q3";

/* -------- Telegram Credentials -------- */

String botToken = "8611288978:AAEvSUNFUsYq_-FcTbT_E25aVVAFXyyCn6I";

String chatID1 = "123456789";     // Your Chat ID

String chatID2 = "987654321";     // Emergency Contact Chat ID

// MPU6050

Adafruit_MPU6050 mpu;

// Thresholds

const float impactThreshold = 2.0;  // G-force threshold for impact

const float tiltThreshold = 45.0;   // Tilt angle threshold in degrees

void setup() {

Serial.begin(115200);

Wire.begin();

// Connect to Wi-Fi

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi Connected!");

// Initialize MPU6050

if (!mpu.begin()) {

Serial.println("MPU6050 not found!");

while (1) delay(10);

}

Serial.println("MPU6050 Initialized");

mpu.setAccelerometerRange(MPU6050_RANGE_8_G);

mpu.setGyroRange(MPU6050_RANGE_500_DEG);

}

void loop() {

sensors_event_t a, g, temp;

mpu.getEvent(&a, &g, &temp);

// Calculate total acceleration in G

float accTotal = sqrt(a.acceleration.x * a.acceleration.x +

a.acceleration.y * a.acceleration.y +

a.acceleration.z * a.acceleration.z) / 9.81;

// Calculate tilt angle (simplified)

float tiltAngle = acos(a.acceleration.z / (sqrt(a.acceleration.x*a.acceleration.x + a.acceleration.y*a.acceleration.y + a.acceleration.z*a.acceleration.z))) * 180 / PI;

Serial.print("Acc Total: "); Serial.print(accTotal);

Serial.print(" G, Tilt: "); Serial.println(tiltAngle);

// Check for accident

if (accTotal > impactThreshold && tiltAngle > tiltThreshold) {

Serial.println("Accident Detected! Sending Telegram Alert...");

sendTelegramAlert();

delay(10000); // Wait 10s before checking again to avoid multiple alerts

}

delay(200); // Loop delay

}

void sendTelegramAlert() {

String message = "⚠️ Accident Alert! Helmet impact detected. Please check immediately.";

// Send to first contact

sendMessage(chatID1, message);

// Send to second contact

sendMessage(chatID2, message);

}

void sendMessage(String chatID, String text) {

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

String url = "https://api.telegram.org/bot" + botToken + "/sendMessage?chat_id=" + chatID + "&text=" + text;

http.begin(url);

int httpResponseCode = http.GET();

if (httpResponseCode > 0) {

Serial.print("Message sent to "); Serial.println(chatID);

} else {

Serial.print("Error sending message: "); Serial.println(httpResponseCode);

}

http.end();

} else {

Serial.println("WiFi Disconnected!");

}

}